PROJECT 29 — SMART IRRIGATION SYSTEM

Enclosure Design Report

| Name | Index No. | Course | Date |
| --- | --- | --- | --- |
| Calvin | 8985323 | Embedded Systems | 11 June 2026 |

1.  Design Overview

The enclosure is a benchtop instrument case that houses the complete Smart Irrigation controller. The display and all user controls sit on the front panel for easy reading and operation, while the ESP32, DHT11 and relay module mount on internal standoffs above a dedicated battery bay. A removable lid gives full access for assembly and debugging. This layout was chosen over a sealed weatherproof box because the unit is operated and graded on a desk, and every component must remain visible and serviceable.

2.  Overall Dimensions

All dimensions are stated in centimetres (cm), as required.

| Parameter | Value (cm) |
| --- | --- |
| Overall width | 15.0 |
| Overall depth | 12.0 |
| Overall height | 6.0 |
| Wall thickness | 0.3 |
| Corner radius | 0.6 |
| Lid clearance | 0.04 |

3.  Dimensioned Drawing

Figure 1 — Top view (internal layout) and front view (panel). Dimensions in cm.

4.  Front Panel Layout

| Feature | Size (cm) | Qty |
| --- | --- | --- |
| LCD 20x4 window | 8.0 x 3.0 | 1 |
| Tactile buttons | Ø1.2 (1.5 spacing) | 3 |
| Indicator LEDs | Ø0.5 | 2 |
| SPDT power switch | Ø1.3 | 1 |
| Pump output jack | Ø1.2 | 1 |

5.  Internal Layout & Component Mounting

Every component from the Project 29 brief maps to a physical feature of the enclosure:

| Component | Mounting / feature |
| --- | --- |
| ESP32 dev module | 4 standoffs, left bay |
| DHT11 sensor | 2 standoffs, centre |
| LDR module | left-side sensor port |
| Soil-sensor wiring | left-side sensor ports |
| Relay module (2-ch) | 4 standoffs, right bay |
| LCD 20x4 + I2C | 8 x 3 cm front window |
| Tactile buttons | 3 x Ø1.2 cm holes @ 1.5 cm |
| Indicator LEDs | 2 x Ø0.5 cm holes |
| SPDT switch | Ø1.3 cm hole |
| Li-Ion battery + case | 7 x 3 cm battery bay |
| Water pump output | Ø1.2 cm jack + cable gland |

6.  Ports, Access & Ventilation

| Location | Feature |
| --- | --- |
| Left side | 3 sensor-port slots (soil probe + LDR wiring) |
| Right side | Cable gland Ø1.2 cm + 4 ventilation slots |
| Top | Removable lid, 4 corner screws (self-locating lip) |

7.  Materials & Manufacturing

The enclosure is designed for FDM 3D printing in PLA or ABS with 0.3 cm walls. The body prints open-side-up with no supports; the lid prints flat. The four corner bosses accept M2.5 self-tapping screws to secure the lid. Rounded R0.6 cm corners improve strength and finish.

8.  Rendered Assembly

Figure 2 — Assembled concept render (grayscale) showing the populated panel.

9.  CAD Files Delivered

| File | Purpose |
| --- | --- |
| irrigation_body_cm.step | Import into OnShape (centimetre units) |
| irrigation_lid_cm.step | Lid — import into OnShape |
| irrigation_body_mm.step / lid | Millimetre backup for other CAD tools |
| irrigation_body.stl / lid | 3D printing (slicer) |
| build_final_cm.py | Editable parametric source (all cm parameters) |

10.  Model Generation & Verification

The geometry was produced from a parametric CadQuery (Python) script in which every dimension is a named centimetre parameter. The STEP files were verified by re-importing and measuring the bounding box: the centimetre file imports at exactly 15.0 x 12.0 x 6.0 cm, confirming the unit and scale are correct before any CAD or print work.