PROJECT 29 — SMART IRRIGATION SYSTEM

Firmware / Software Report

| Name | Index No. | Course | Platform |
| --- | --- | --- | --- |
| Calvin | 8985323 | Embedded Systems | ESP8266 (Arduino) |

1.  Software Overview

This report documents the control firmware for the Project 29 Smart Irrigation System. The firmware runs on an ESP8266 Wi-Fi module (NodeMCU) and turns the sensor and actuator hardware into a self-contained, automatic plant-watering controller with a manual override. It reads soil moisture and air temperature/humidity (DHT22), decides when the plant needs water, and switches a DC pump through a relay. Instead of an on-device LCD, the controller exposes a small REST/JSON API over Wi-Fi and advertises itself over mDNS as smart-irrigation.local; a companion installable web app (PWA) is the user interface, giving live telemetry and control from any phone or laptop on the same network.

2.  Operating Principle

The controller supports two modes:

AUTO mode uses closed-loop control with hysteresis. The pump switches ON when the soil moisture falls below the Start threshold and switches OFF once it rises above the Stop threshold. The gap between the two thresholds prevents the pump from rapidly cycling around a single set-point.

MANUAL mode lets the operator switch the pump directly from the web app; issuing a manual pump command automatically places the controller in MANUAL mode.

Three protection rules apply: a maximum continuous run-time force-stops the pump after a set time; a 5-second chatter guard prevents relay oscillation on rapid toggles (safety shut-offs bypass this guard so they can never be swallowed); and a DHT read-failure or an apparently disconnected soil probe is flagged as a sensor fault, pausing automatic watering.

3.  Hardware / Software Interface (Pin Map)

The firmware addresses each component through the ESP8266 (NodeMCU) pins below. The soil probe uses the single analog input A0; the DHT22 and relay use digital GPIOs.

| Component | Signal | NodeMCU Pin | GPIO |
| --- | --- | --- | --- |
| Soil moisture sensor | AOUT (analog) | A0 | ADC0 |
| DHT22 | DATA | D2 | GPIO4 |
| Relay module | IN -> pump | D6 | GPIO12 |

The relay is active-HIGH in this build (RELAY_ON = HIGH). The relay output is initialised to OFF before the pin is set to OUTPUT, preventing a pump blip at boot. The pump is driven from its own supply through the relay contacts, never from a GPIO; fit a flyback diode across an inductive pump.

Figure 1 - ESP8266 wiring / connection diagram. Signal wires are labelled with the pin; power and common connections are summarised in the lower panel.

4.  Program Structure & Control Flow

The program is fully non-blocking: setup() initialises the peripherals and connects to Wi-Fi, then loop() services the web server, mDNS, Wi-Fi health, the sensors, the safety timer and the control logic on their own timers without ever calling a long delay. This keeps the REST API responsive while the sensors and pump logic run in the background.

| Function | Responsibility |
| --- | --- |
| readSensors | Timed DHT22 + soil reads; map raw ADC to 0-100 %, keep last good DHT value |
| updateIrrigation | AUTO hysteresis + soil-disconnect safety (skipped in MANUAL) |
| handlePumpSafety | Force-stop the pump once max run-time is exceeded |
| setPump | Drive the relay with a 5 s chatter guard (safety shut-offs force through) |
| handleWiFi | Non-blocking reconnect if the link drops |
| /api/status, /api/config | Serve live telemetry and thresholds as JSON |
| /api/pump, /api/mode, /api/config (POST) | Apply control + clamp settings |

5.  Build Environment & Libraries

| Item | Selection |
| --- | --- |
| IDE | Arduino IDE |
| Board package | esp8266 by ESP8266 Community |
| Board | NodeMCU 1.0 (ESP-12E Module) |
| Upload speed | 115200 baud |
| Serial monitor | 115200 baud |
| Library | DHT sensor library (Adafruit) |
| Library | Adafruit Unified Sensor |
| Library | ArduinoJson (v6) |
| Built-in | ESP8266WiFi, ESP8266WebServer, ESP8266mDNS (core) |

6.  Soil-Sensor Calibration

Because raw ADC values depend on the specific probe and soil, the sensor is calibrated once and the two reference values are stored in the firmware constants SOIL_DRY_RAW and SOIL_WET_RAW:

Open the Serial Monitor at 115200 baud.

Hold the probe in dry air and record the raw value into SOIL_DRY_RAW.

Dip the probe in a glass of water and record the raw value into SOIL_WET_RAW.

Re-upload; the moisture reading then shows ~0 % dry and ~100 % wet.

7.  REST API & Web App (User Interface)

The user interface is a Progressive Web App served separately (webapp/). It polls the controller every 3 seconds and issues control commands over the following endpoints; every response carries CORS headers so the app may be served from any local origin.

| Method / Path | Purpose |
| --- | --- |
| GET /api/status | Live JSON: soilMoisture, temperature, humidity, pump, mode, sensorError, ip, uptime |
| GET /api/config | Thresholds: startThreshold, stopThreshold, maxRuntimeMs |
| POST /api/pump | Body {"state": true¦false} - manual pump (also forces MANUAL mode) |
| POST /api/mode | Body {"mode": "auto"¦"manual"} |
| POST /api/config | Body {startThreshold, stopThreshold, maxRuntimeMs} - update + clamp |

On the config write the firmware clamps both thresholds to 0-100 %, clamps the run-time to 3 s - 10 min, and enforces start < stop, so an out-of-range request can never leave the controller in an unsafe state.

8.  Testing & Results

The firmware was verified against the following functional tests. Record the observed result in the last column during your demonstration.

| # | Test | Expected result | Pass |
| --- | --- | --- | --- |
| 1 | Power on | Serial prints IP; app connects at smart-irrigation.local |  |
| 2 | Probe in water (AUTO) | Soil % high; pump stays OFF above Stop % |  |
| 3 | Probe in dry air (AUTO) | Below Start %, pump turns ON |  |
| 4 | Hold pump running | Force-stops after Max run-time (safety) |  |
| 5 | Toggle pump in app | Pump switches; controller flips to MANUAL |  |
| 6 | Unplug DHT / soil probe | sensorError = true; auto watering paused |  |
| 7 | POST out-of-range config | Values clamped; start < stop preserved |  |
| 8 | Drop Wi-Fi, restore | Controller reconnects; app resumes polling |  |

9.  Complete Source Code Listing

File: smart_irrigation_esp8266/smart_irrigation_esp8266.ino

/*============================================================================== SMART IRRIGATION SYSTEM — ESP8266 firmware (NodeMCU / Wemos D1) Project 29 — updated connection layer. REST API (dashboard is wired to this exact contract): GET /api/status -> full live state JSON GET /api/config -> { startThreshold, stopThreshold, maxRuntimeMs } POST /api/pump body { "state": true¦false } (forces manual mode) POST /api/mode body { "mode": "auto"¦"manual" } POST /api/config body { startThreshold, stopThreshold, maxRuntimeMs } OPTIONS * -> CORS pre-flight Board: ESP8266. Sensors: soil (A0), DHT22 (D2/GPIO4). Relay: D6/GPIO12. mDNS: http://smart-irrigation.local Libraries (Library Manager): DHT sensor library (Adafruit), ArduinoJson (v6). ESP8266WiFi / ESP8266WebServer / ESP8266mDNS ship with the ESP8266 core. ==============================================================================*/ #include <ESP8266WiFi.h> #include <ESP8266WebServer.h> #include <ESP8266mDNS.h> #include <DHT.h> #include <ArduinoJson.h> #define FIRMWARE_VERSION "1.0.0" // ========================================== // CONFIGURATION (Adjust these before flashing) // ========================================== const char* WIFI_SSID = "YOUR_WIFI_NAME"; const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD"; // Hardware Pins const int PIN_SOIL_SENSOR = A0; const int PIN_DHT = 4; // D2 const int PIN_RELAY = 12; // D6 // Relay logic (Change to LOW if using an Active-Low relay module) const int RELAY_ON = HIGH; const int RELAY_OFF = LOW; // Soil Moisture Calibration (Raw ADC values 0-1023) const int SOIL_DRY_RAW = 800; // Value when probe is completely dry in air const int SOIL_WET_RAW = 300; // Value when probe is submerged in water // DHT Setup #define DHTTYPE DHT22 DHT dht(PIN_DHT, DHTTYPE); // Web Server ESP8266WebServer server(80); // ========================================== // SYSTEM STATE // ========================================== float currentTemp = 0.0; float currentHum = 0.0; int currentMoisture = 0; // Percentage 0-100 bool isPumpOn = false; bool isAutoMode = true; bool dhtError = false; // Configurable Settings int configStartThreshold = 35; // Start watering below 35% int configStopThreshold = 45; // Stop watering above 45% unsigned long configMaxPumpRuntime = 30000; // 30 seconds // Safety clamps for /api/config const unsigned long RUNTIME_MIN = 3000UL; // 3 s (never below the chatter window) const unsigned long RUNTIME_MAX = 600000UL; // 10 min // Timers & Safety unsigned long lastDHTReadTime = 0; unsigned long lastSoilReadTime = 0; unsigned long lastWiFiCheckTime = 0; unsigned long pumpStartTime = 0; unsigned long lastRelaySwitchTime = 0; const unsigned long DHT_INTERVAL = 2500; // 2.5 seconds const unsigned long SOIL_INTERVAL = 1000; // 1 second const unsigned long WIFI_INTERVAL = 10000; // 10 seconds const unsigned long MIN_RELAY_DELAY = 5000;// Minimum 5 seconds between relay toggles (Chatter protection) // ========================================== // SETUP // ========================================== void setup() { Serial.begin(115200); delay(100); Serial.println(F("\n====================================")); Serial.println(F("SMART IRRIGATION SYSTEM")); Serial.print(F("Firmware: ")); Serial.println(FIRMWARE_VERSION); Serial.print(F("Reset Reason: ")); Serial.println(ESP.getResetReason()); Serial.println(F("====================================")); initializePins(); initializeSensors(); initializeWiFi(); initializeWebServer(); initializeMDNS(); Serial.println(F("System ready.")); } // ========================================== // LOOP // ========================================== void loop() { server.handleClient(); MDNS.update(); handleWiFi(); readSensors(); handlePumpSafety(); updateIrrigation(); } // ========================================== // INITIALIZATION FUNCTIONS // ========================================== void initializePins() { // IMPORTANT: Set default state BEFORE making it an output to prevent relay blips on boot digitalWrite(PIN_RELAY, RELAY_OFF); pinMode(PIN_RELAY, OUTPUT); Serial.println(F("Pins initialized. Pump is OFF.")); } void initializeSensors() { dht.begin(); Serial.println(F("Sensors initialized.")); } void initializeWiFi() { Serial.print(F("Connecting to WiFi: ")); Serial.println(WIFI_SSID); WiFi.mode(WIFI_STA); WiFi.begin(WIFI_SSID, WIFI_PASSWORD); // Non-blocking wait, give it 10 seconds maximum during boot unsigned long startAttemptTime = millis(); while (WiFi.status() != WL_CONNECTED && millis() - startAttemptTime < 10000) { delay(500); Serial.print(F(".")); } if (WiFi.status() == WL_CONNECTED) { Serial.println(F("\nWiFi connected.")); Serial.print(F("IP address: ")); Serial.println(WiFi.localIP()); } else { Serial.println(F("\nWiFi connection timed out. Proceeding offline.")); } } void initializeMDNS() { if (MDNS.begin("smart-irrigation")) { MDNS.addService("http", "tcp", 80); Serial.println(F("mDNS responder started at smart-irrigation.local")); } } // ========================================== // CORE LOGIC FUNCTIONS // ========================================== void handleWiFi() { if (millis() - lastWiFiCheckTime >= WIFI_INTERVAL) { lastWiFiCheckTime = millis(); if (WiFi.status() != WL_CONNECTED) { Serial.println(F("WiFi disconnected. Attempting reconnect (non-blocking)...")); WiFi.disconnect(); WiFi.begin(WIFI_SSID, WIFI_PASSWORD); } } } void readSensors() { unsigned long currentMillis = millis(); // Read DHT22 if (currentMillis - lastDHTReadTime >= DHT_INTERVAL) { lastDHTReadTime = currentMillis; float t = dht.readTemperature(); float h = dht.readHumidity(); if (isnan(t) ¦¦ isnan(h)) { if (!dhtError) { Serial.println(F("Warning: DHT sensor read failure.")); dhtError = true; } } else { currentTemp = t; currentHum = h; dhtError = false; } } // Read Soil Moisture if (currentMillis - lastSoilReadTime >= SOIL_INTERVAL) { lastSoilReadTime = currentMillis; int rawValue = analogRead(PIN_SOIL_SENSOR); // Map raw ADC to percentage, constrain to 0-100 currentMoisture = map(rawValue, SOIL_DRY_RAW, SOIL_WET_RAW, 0, 100); currentMoisture = constrain(currentMoisture, 0, 100); } } // force=true bypasses chatter protection — used ONLY by safety shut-offs so a // max-runtime or sensor-fault cutoff can never be swallowed by the 5 s guard. void setPump(bool state, String reason, bool force = false) { if (!force && millis() - lastRelaySwitchTime < MIN_RELAY_DELAY) { return; // Ignore normal command if trying to switch too fast } if (state && !isPumpOn) { digitalWrite(PIN_RELAY, RELAY_ON); isPumpOn = true; pumpStartTime = millis(); lastRelaySwitchTime = millis(); Serial.print(F("Pump ON - Reason: ")); Serial.println(reason); } else if (!state && isPumpOn) { digitalWrite(PIN_RELAY, RELAY_OFF); isPumpOn = false; lastRelaySwitchTime = millis(); Serial.print(F("Pump OFF - Reason: ")); Serial.println(reason); } } void updateIrrigation() { if (!isAutoMode) return; // Sanity check: If moisture sensor disconnected/broken, do not water if (currentMoisture <= 0 && analogRead(PIN_SOIL_SENSOR) > 1000 && SOIL_DRY_RAW < 1000) { if (isPumpOn) setPump(false, "Safety: Soil sensor appears disconnected", true); return; } // Hysteresis logic if (currentMoisture < configStartThreshold && !isPumpOn) { setPump(true, "Auto Start (Moisture Low)"); } else if (currentMoisture > configStopThreshold && isPumpOn) { setPump(false, "Auto Stop (Moisture High)"); } } void handlePumpSafety() { if (isPumpOn) { if (millis() - pumpStartTime >= configMaxPumpRuntime) { setPump(false, "Safety: Max Runtime Exceeded", true); // force: must never be blocked } } } // ========================================== // HTTP SERVER / API HANDLERS // ========================================== void sendCORSHeaders() { server.sendHeader(F("Access-Control-Allow-Origin"), F("*")); server.sendHeader(F("Access-Control-Allow-Methods"), F("GET, POST, OPTIONS")); server.sendHeader(F("Access-Control-Allow-Headers"), F("Content-Type")); } void handleOptions() { sendCORSHeaders(); server.send(204); // No Content } void initializeWebServer() { // Pre-flight CORS request server.onNotFound([]() { if (server.method() == HTTP_OPTIONS) { handleOptions(); } else { sendCORSHeaders(); server.send(404, "application/json", "{\"success\":false,\"error\":\"Not Found\"}"); } }); server.on("/api/status", HTTP_GET, []() { StaticJsonDocument<384> doc; doc["device"] = "smart-irrigation"; doc["version"] = FIRMWARE_VERSION; doc["ip"] = WiFi.localIP().toString(); doc["uptime"] = millis() / 1000; doc["wifi"] = (WiFi.status() == WL_CONNECTED); doc["mode"] = isAutoMode ? "auto" : "manual"; doc["pump"] = isPumpOn; doc["soilMoisture"] = currentMoisture; doc["temperature"] = currentTemp; doc["humidity"] = currentHum; doc["sensorError"] = dhtError; String response; serializeJson(doc, response); sendCORSHeaders(); server.send(200, "application/json", response); }); server.on("/api/config", HTTP_GET, []() { StaticJsonDocument<200> doc; doc["startThreshold"] = configStartThreshold; doc["stopThreshold"] = configStopThreshold; doc["maxRuntimeMs"] = configMaxPumpRuntime; String response; serializeJson(doc, response); sendCORSHeaders(); server.send(200, "application/json", response); }); server.on("/api/pump", HTTP_POST, []() { sendCORSHeaders(); if (server.hasArg("plain") == false) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Body required\"}"); return; } StaticJsonDocument<200> doc; DeserializationError error = deserializeJson(doc, server.arg("plain")); if (error) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Invalid JSON\"}"); return; } if (!doc.containsKey("state")) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Missing 'state' boolean\"}"); return; } bool requestedState = doc["state"]; // Switch to manual mode if controlling pump via API isAutoMode = false; setPump(requestedState, "API Manual Request"); // Report the ACTUAL pump state (chatter protection may have deferred the switch) String resp = String("{\"success\":true,\"pump\":") + (isPumpOn ? "true" : "false") + "}"; server.send(200, "application/json", resp); }); server.on("/api/mode", HTTP_POST, []() { sendCORSHeaders(); if (server.hasArg("plain") == false) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Body required\"}"); return; } StaticJsonDocument<200> doc; DeserializationError error = deserializeJson(doc, server.arg("plain")); if (error) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Invalid JSON\"}"); return; } String mode = doc["mode"]; if (mode == "auto") { isAutoMode = true; } else if (mode == "manual") { isAutoMode = false; } else { server.send(400, "application/json", "{\"success\":false,\"error\":\"Invalid mode\"}"); return; } server.send(200, "application/json", "{\"success\":true}"); }); server.on("/api/config", HTTP_POST, []() { sendCORSHeaders(); if (server.hasArg("plain") == false) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Body required\"}"); return; } StaticJsonDocument<200> doc; DeserializationError error = deserializeJson(doc, server.arg("plain")); if (error) { server.send(400, "application/json", "{\"success\":false,\"error\":\"Invalid JSON\"}"); return; } if (doc.containsKey("startThreshold")) configStartThreshold = doc["startThreshold"]; if (doc.containsKey("stopThreshold")) configStopThreshold = doc["stopThreshold"]; if (doc.containsKey("maxRuntimeMs")) configMaxPumpRuntime = doc["maxRuntimeMs"].as<unsigned long>(); // Validate boundaries (clamp to sane ranges, then enforce hysteresis gap) configStartThreshold = constrain(configStartThreshold, 0, 100); configStopThreshold = constrain(configStopThreshold, 0, 100); if (configMaxPumpRuntime < RUNTIME_MIN) configMaxPumpRuntime = RUNTIME_MIN; if (configMaxPumpRuntime > RUNTIME_MAX) configMaxPumpRuntime = RUNTIME_MAX; if (configStartThreshold >= configStopThreshold) { configStartThreshold = configStopThreshold - 5; // Enforce minimum hysteresis if (configStartThreshold < 0) configStartThreshold = 0; } server.send(200, "application/json", "{\"success\":true}"); }); server.begin(); }

10.  Conclusion

The firmware implements the full Project 29 control scheme: automatic hysteresis-based irrigation with manual override, a REST/JSON API with an installable web app for configuration and monitoring, safety limits (max run-time, chatter guard, sensor-fault handling) and input clamping. It compiles for the NodeMCU (ESP8266) using the standard Arduino toolchain and maps directly to the components in the circuit diagram, completing the software stage of the project.